School of Electronics Engineering (SENSE) B. Tech – Electronics & Computer Engineering BECE403E – EMBEDDED SYSTEM DESIGN LAB RECORD (L15+L16) Submitted By 21BLC1228 – Mayukh Ray Submitted To Dr. Manoj Kumar Rajagopal DATE: 29/02/2024 Slot: L15+L16 Date: 29/2/2024 LAB – 08: Working with SPI AIM: Implement and verify the logic on the STM32 Nucleo-64 board using Keil Studio Cloud IDE. Software Required: ARM Keil Studio (Mbed Online Compiler) Hardware Required: Micro USB cable, NUCLEO64-STM32L152 Board, LEDs, Jumper Wires (M-F and M-M), Breadboard Procedure: - Go to ARM Keil Studio (https://studio.keil.arm.com) and log in - Select File → New → Mbed Project - Click the Example project drop-down list and select “mbed2-example-blinky” - In Project name field, provide the name of the new project and click Add project - Double click on the “main.cpp” file from the newly created project folder - Modify the code in the editor window as per the logic of your application - Check for any errors in the program under the “Problems” tab of the panels window - If no errors, connect the Nucleo Board to the computer using Micro USB Cable - Click Play icon (Run project) to upload and start the code execution on the board. PROGRAM: Lab Task 1: Write a program to implement a SPI communication between two Nucleo boards. Configure one of the Nucleo as master and other as slave. Establish a SPI communication between master and slave display each key press on the master’s Teraterm to the slave Teraterm terminal. Code: Program (Master) #include “mbed.h” SPI spi(PB_15,PB_14,PB_13); DigitalOut cs(PB_12); Serial pc(USBTX,USBRX); int main(){ char send_val; pc.printf(“Press any key to start…\n”); while(1){ send_val=pc.getc(); pc.printf(“%c”,send_val); cs=0; spi.write(send_val); cs=1; wait(0.01); } } Program (Slave) #include “mbed.h” SPISlave spi(PB_15,PB_14,PB_13,PB_12); Serial pc(USBTX,USBRX); Char recd_val; int main(){ pc.printf(“Received word is…\n”); while(1){ if(spi.receive()){ recd_val=spi.read(); pc.printf(“%c”,recd_val); } } } Output: Output Verification: Lab Task 2: Write a program to implement a SPI communication between two Nucleo boards. Configure one of the Nucleo as master and other as slave. Both Nucleo are attached with a LED & a push button separately. Master LED can be controlled by using slave Nucleo’s push button and slave Nucleo’s LED can be controlled by master Nucleo’s push button using SPI communication protocol. Code: Program (Master) #include “mbed.h” SPI ser_port(PB_15,PB_14,PB_13); Digitalout led(PC_8); DigitalIn switch_ip(PC_4); DigitalOut cs(PB_12); char switch_word; char recd_val; int main() { while(1) { switch_word=0xa0; if(switch_ip==1) switch_word=switch_word¦0x01; cs=0; recd_val=ser_port.write(switch_word); cs=1; wait(0.01); led=0; recd_val=recd_val&0x01; if(recd_val==1) led=1; } } Program (Slave) #include “mbed.h” SPISlave ser_port(PB_15,PB_14,PB_13,PB_12); DigitalOut led(PC_8); DigitalIn switch_ip(PC_4); char recd_val; int main() { while(1) { switch_word=0xa0; if(switch_ip==1) switch_word=switch_word¦0x01; if(ser_port.receive()) { recd_val=ser_port.read(); ser_port.reply(switch_word); } led=0; recd_val=recd_val&0x01; if(recd_val==1) led=1; } } Output: Output Verification: INFERENCE: - We need to identify the pin connected to the LED. - Write the program to toggle the state of the LED between ON and OFF in a loop. - Compile and upload the code to the microcontroller. RESULT: Lab-1: Hence, we were able to implement a SPI communication between two Nucleo boards. Configured one of the Nucleo as master and other as slave. Establish a SPI communication between master and slave display each key press on the master’s Teraterm to the slave Teraterm terminal. Lab-2: Hence, we were able to implement a SPI communication between two Nucleo boards. Configured one of the Nucleo as master and other as slave. Both Nucleo are attached with a LED & a push button separately. Master LED can be controlled by using slave Nucleo’s push button and slave Nucleo’s LED can be controlled by master Nucleo’s push button using SPI communication protocol.